SYSTEM IDENTIFICATION

参考视频：研究生课程｜系统辨识全 77 讲：从测量数据到数学模型（UCCS ECE5560 · 中文字幕）_哔哩哔哩_bilibili

华中科技大学 人工智能与自动化学院 人工智能启明2501班 宋润

目录

1.1背景概念	2

1.2 吹风机Dryer实例	4

2.1 Continuous-time linear time-invariant systems	5

# 1.1背景概念

10.16.2026

- Box

- White box(transparent透明box):the dynamics of a system for which a lot of information is known a priori. Unknown parameters can be measured directly.

- Black box:

- Gray box(translucent 半透明box):

- Don’t estimate what you know

- This course looks at input-output models. Attach sensors to those internal variables, and then they would become outputs.

- The second “collect data” has different noise, to avoid that the model fits noise really well but is awful

Validate验证

- Some issues with experiment design:

- Open loop or closed loop开环/闭环辨识

- What is the input sequence?

- Frequency content , Actuator执行器 limits

- What sampling rate and data length (memory)?

- Depending a lot on the frequency response频率响应

- Date Filtering筛选:Must deal with drifts漂移 , biases偏差 , outliers异常值 ; attenuate抑制 noise.

- Some issues with model structure:

- NON-PARAMETRIC: Frequency response plot, or impulse脉冲 response (simplest but poorest , most computation) (参数数量不确定)

- PARAMETRIC:

- System class: linear or nonlinear

- Model size: number of pole极点and zeros of system transfer function and disturbance transfer function. Also, how many delays between input and output?

- Some issues with fitting the model:

- big tradeoff between accuracy and ease of solution

- Significant degree of user input required

- Some issues with validation:

- Different date sets (split it in half if you have only one set)

- Time- and frequency-domain analysis

- Stochastic随机的 analysis of the residual残差error

- Does result imply that we should make changes to:

- Experiment (input sequence),

- Model choice (order, type.,...),

- Objective function for fit,

- Optimization procedure

# 1.2 吹风机Dryer实例

- 10.17.2026

- PRBS : Pseudo-random binary sequence伪随机二进制序列.

- Bode plot : A Bode plot shows frequency response with two logarithmic对数的 curves: magnitude幅值 (gain in增益 dB vs frequency) and phase相位 (degrees vs frequency).

- Inverse逆

# 2.1 Continuous-time linear time-invariant systems

10.18.2026

- linear time-invariant (LTI)线性时不变

- Test a system if it is a LTI system

A time-invariant system will have y2(t) = y1(t - dt) for all u1(t) and dt.

- “delay operator”

y(t)=u(t−λ) , λ≥0.

- LINEAR

For linear systems, if u1(t)↦y1(t) and u2(t)↦y2(t), the

u3(t)=αu1(t)+βu2(t)↦y3(t)=αy1(t)+βy2(t)

- LTI system has an impulse response冲激响应 , which entirely characterizes the system’s dynamics

- The Fourier transform of the impulse response is the frequency response.

- The Laplace transform of the impulse response is the transfer function.

- Lumped LTI systems can also be represented in state-space form.

- Lumped 集总（反义词是分布Distributed）